# Circuits and Systems: ECE 3364

Homework 1

This homework assignment includes problems from the textbook, which is Electric Circuits, by Nilsson and Riedel, 12th Edition.  Please submit your solutions with all necessary work, through Canvas, by the due date.

Chapter 10: Problems 10.6, 10.8, 10.10, 10.24, 10.26

# Note on units in Nilsson and Riedel textbook

In the Nilsson and Riedel textbook they are generally good about showing units, but there are some exceptions.  Among the exceptions are that they assume that time is always given in units of [seconds] or [s].  So, when in Problem 10.6, when they tell us that

what they mean is that

Showing the (t), which indicates the independent variable, is optional.  The current units can be placed next to number it goes with, or at the end of the expression.  I will show all units, but you are not required to show units where Nilsson and Reidel do not show those units.  I place my units within square brackets, but you do not need to do this, either.

The second common exception is when labeling the units on certain dependent sources, specifically voltage-dependent current sources, and current-dependent voltage sources.  This can be seen in Problem 10.8.  The current dependent voltage source is that circuit is labeled

And they assume that the units are [Volts/Ampere], or [W], so we would label that source as

Again, I will show all units, but you are not required to show units where Nilsson and Reidel do not show those units.

In Problem 10.10, the line capacitor is labeled in an inappropriate way, in Figure P10.10.  First, the diagram shows units for a variable.  We do not do that; please ignore the units on the variable for the line capacitor.  Next, the capacitor impedance is shown as -jX.  The capacitor has a negative reactance, but we use X for reactance.  Therefore, this is misleading, since the label is invariably the impedance of that component.  In my solution, I solved for X as defined in the diagram, but gave my answer for reactance as the negative of my solutions for X.  I believe that this problem should be modified.

Selected Numerical Solutions:

10.6.  990[mW]

10.8.  2.125[kW]

10.10.  a)  Reactance of the line is -10[W] or -110[W].  b)  For reactance of the line equal to 
-10[W], the Qdel.by.source = 25[kVAR], which checks.  For reactance of the line equal to 
-110[W], the Qdel.by.source = -25[kVAR], which checks.

10.24.  a)  For Load 1, pf = 0.9046 lagging, and rf = 0.4263.  For Load 2, pf = 0.4346 leading, and rf = -0.9006.  For Load 3, pf = 0.5740 leading, and rf = -0.81887.  b)  For composite load, 
pf = 0.9393 lagging, and rf = 0.34305.

20.26.  a)  Pdel.by.Vg1 = 4200[W], Qdel.by.Vg1 = 1200[VAR], Pdel.by.Vg2 = 3600[W], 
Qdel.by.Vg2 = -300[VAR].  b)  Pdel.by.sources = 7800[W] = Pabs.by.loads, which checks.  
Qdel.by.sources = 900[VAR] = Qabs.by.loads, which checks.