# In Class Practice

# #07: Special Purpose Registers

Give all answers in hexadecimal. Refer to the code provided for this homework for all answers.

- At what address in ROM is the instruction on LINE 46 (MOV R0, #0) stored at

Answer: ________________________________

- What is the machine code for the following instruction? BX LR

Answer: ________________________________

- What is the machine code for the following instruction? Line 145: UDIV R0, R0, R5

Answer: ________________________________

- What is the value (address) stored in the LR after line 59 (BL GetSum) is executed?

Answer: ________________________________

- What is the value (address) stored in the PC after line 99 (BX LR) is executed?

Answer: ________________________________

- What is the value (address) stored in the SP before line 54 (PUSH {R5, R6}) is executed?

.

Answer: ________________________________

- What is the value (address) stored in the SP after line 54 (PUSH {R5, R6}) is executed?

.

Answer: ________________________________

- What is the value (address) stored in the SP after line 74 (POP {R5, R6}) is executed?

Answer: ________________________________

- What value (address) stored in the LR lets you know that no subroutines have been called?

Answer: ________________________________

- What value (address) stored in the SP lets you know that the stack is empty?

Answer: ________________________________