In arithmetic, how do you “activate” the flags in Assembly? Answer: Add the suffix “S” to arithmetic operations (e.g., ADDS, SUBS). This updates the APSR condition flags (N, Z, C, V). What condition flags are affected by a signed addition? Answer: V (overflow), N (negative), and Z (zero). Unsigned Subtraction — what flags are affected? Answer: C (carry/borrow), plus N and Z. When is C set or cleared? Answer: C set when unsigned addition overflows OR unsigned subtraction is good (no borrow). C cleared when unsigned addition is good OR unsigned subtraction borrows. When is V set? Answer: When a signed result crosses the 0x7FFF_FFFF ↔ 0x8000_0000 boundary (overflow). When is N set? Answer: When the result is negative (MSB = 1). What makes an unsigned addition “bad”?Answer: When the result wraps around 0 (overflow).C bit = 1 when this happens. Example: $999 + 200 → 199 (crossed zero boundary). What makes unsigned subtraction “bad”?Answer: When subtraction crosses 0 (borrow).C bit = 0 when this happens. Example: $100 − $300 → $800 (wraps around). What makes signed addition “bad”? Answer: When the result crosses ±2ⁿ⁻¹ boundary (overflow). V set when “bad” V clear when “good.” What makes signed subtraction “bad”? Answer: Same as signed addition — crosses ±2ⁿ⁻¹ boundary. V set when “bad,” V clear when “good.” How do unsigned and signed arithmetic differ regarding flags? Answer Unsigned uses C (carry/borrow) Signed uses V (overflow). Both also update N and Z. 0x7FFF_FFFF + 0x2 Flag set: N, V, 0x7FFF_FFFF – 0x200:C, 0x100 + 0x500: none, 0x100 – 0x300:N, 0x7FFF_FFF + 0x1: C, 0x7FFF_FFF + 0x1:N,V Q: Format for branch statements? A: B{cond} label; BL{cond} label (saves LR). Q: Check if number positive, else subtract 5 A: CMP R0,#0; BGT add5; SUB R0,R0,#5; B done; add5: ADD R0,R0,#5; done: Q: If number<10, call subroutine A else B A: CMP R0,#10; BLT subA; BL subB; Q: Convert C code if(B!=0)... A: LDRB R0,[B]; CMP R0,#0; BNE nz; BL Bequalzero; B done; nz: BL Bnotzero; done: Q: If (A<-20) or (A>30)A: CMP R0,#-20; BLT isTooLow; CMP R0,#30; BGT isTooHigh; BL isWithinRange; Q: Complex condition B==0&&C;<0A: LDR R1,[B]; LDR R2,[C]; CMP R1,#0; BNE chk; CMP R2,#0; BLT both; B else; both: BL isBZeroAndCNeg; ... Q: What flag(s) do shift/rotate affect? A: C (last bit shifted out), N/Z if suffixed with S. Q: When use ASR vs LSR? A: ASR keeps sign bit for signed; LSR for unsigned. Q: Example: LSL 0x40000000 by 2 A: Result: 0x00000000; N=0, Z=1, C=1, V=0. Q: Example: ASR -4 by 1 A: Result: 0xFFFFFFFE; N=1, Z=0, C=0, V=0. Q: Example: RRX 0x80000000 with C=1 A: Result: 0xC0000000; C cleared. Q: Example: ROR 0x00000003 by 1 A: Result: 0x80000001; C set. Q: Example: LSL 0x40000000 by 1 A: Result: 0x80000000; N=1, Z=0, C=0. Q: Multiply by 4 using shift A: LSL #2 ® Result: 0x40000000. Q: Divide by 8 using shift A: LSR #3 ® Result: 0x10000000. Q: Format for branch statements? A: B{cond} label; BL{cond} label (saves LR). Q: Check if number positive, else subtract 5 A: CMP R0,#0; BGT add5; SUB R0,R0,#5; B done; add5: ADD R0,R0,#5; done: Q: If number<10, call subroutine A else B A: CMP R0,#10; BLT subA; BL subB; Q: Convert C code if(B!=0)... A: LDRB R0,[B]; CMP R0,#0; BNE nz; BL Bequalzero; B done; nz: BL Bnotzero; done: Q: If (A<-20) or (A>30) A: CMP R0,#-20; BLT isTooLow; CMP R0,#30; BGT isTooHigh; BL isWithinRange; Q: Complex condition B==0&&C<0 A: LDR R1,[B]; LDR R2,[C]; CMP R1,#0; BNE chk; CMP R2,#0; BLT both; B else; both: BL isBZeroAndCNeg; chk: CMP R1,#0; BGT either; CMP R2,#0; BGT either; B else; either: BL isEitherPositive; else: BL isBothNegativeOrZero;