## Part A: Multiple Choice (Circle the correct answer) – 20 Questions

- What does the CMP instruction do?

  - Adds two values

  - Subtracts and sets flags – COMPARE subtracts operand 2 from operand 1 and updates NZCV flags

  - Tests bits for zero

  - Compares signs

- Which instruction checks if two values are equal using XOR?

  - CMP

  - TST – checks specific bits

  - TEQ – “test if equal”

  - CMN – only checks sign

- What is the purpose of TST?

  - Compare signs – CMN

  - Test bits for zero – tests if certain bits are 0 where the mask = 1. Result bits = 1 if both data & mask bits = 1; otherwise result bit = 0 (bitwise ANDS & updates NZ)

  - Add values – ADD

  - Branch unconditionally – B

- Which instruction adds two values and sets flags?

  - CMP – subtracts

  - CMN – XOR (exclusive OR – adds)

  - TST – bitwise test for zero

  - TEQ – “test if equal”

- Which flag is affected by comparison instructions?

  - Overflow

  - Zero

  - Carry

  - Negative

- What is the best way to simulate a “for” loop in ARM?

  - CMP + BNE

  - TEQ

  - TST

  - EQU

- What does BNE do?

  - Branch if equal

  - Branch if not equal

  - Branch always

  - Branch if greater

- What does TST R3, #0x0F check? 0x0F = 0000 1111 in binary

  - If R3 equals 0x0F

  - If lower 4 bits of R3 are 0

  - If R3 is negative

  - If R3 is greater than 0x0F

- What instruction saves the return address?

  - STR – “store”

  - BL – “branch linked”

  - BX – “branch extended”

  - LDR – “load”

- What instruction returns from a subroutine?

  - BX LR – returns to address in LR

  - BL

  - CMP

  - STR

- What does PUSH {R4-R7} do?

  - Saves registers – PUSH saves registers into stack

  - Loads values

  - Branches

  - Compares values

- What does EQU do?

  - Loads a value – LDR

  - Defines a constant

  - Compares values – CMP

  - Branches – B

- What instruction sets a bit in a register?

  - ORR – bitwise OR; ORR R0, #2_0101, #2_0010: R0 = #2_0111

  - AND – can be used to clear bits

  - CMP

  - TST

- What instruction clears a bit?

  - BIC – bitwise clear

  - ORR

  - CMP

  - TST

- What is the first step in configuring the system clock?

  - Set GPIO – must activate clock for port first

  - Configure RCC

  - Enable interrupts – unrelated

  - Load SysTick – already configured clock

- What register is used to configure SysTick delay?

  - STRELOAD – 0x00FFFFFF (max 24-bit delay value)

  - RCC

  - GPIO

  - PC

- What does LDR R0, =0x40025000 do?

  - Loads value from memory – LDR R1, [R0]

  - Loads address into R0

  - Stores value – STR R1, [R0]

  - Compares values – load value into R1, then “CMP R1, #0”

- What is the purpose of STR R1, [R0]?

  - Store R1 into memory at R0

  - Load R1 from R0 – LDR R1, [R0]

  - Compare R1 and R0 – CMP R1, R0

  - Branch to R0 – nonsense

- What does MOV R0, #0 do?

  - Load 0 into R0

  - Compare R0 with 0 – CMP R0, #0

  - Store 0 – STR #0, R0

  - Branch to 0 – nonsense

- What is the purpose of CMP R0, #10 followed by BNE loop?

  - Loop if R0 == 10 – must be followed by BEQ loop

  - Loop if R0 != 10

  - Loop if R0 > 10 – must be followed by BGT loop

  - Loop if R0 < 10 – must be followed by BLT loop

## Part B: Fill-in-the-Blank Assembly Labs

Write the missing instruction or value in each blank.

Lab 1: Compare and Branch

MOV R0, #5

CMP R0, #5

BEQ loop     ; Branch if equal (Z=1)

Lab 2: Test Bits

MOV R1, #0xF0

TST R1, #0x0F   ; Test lower 4 bits

Lab 3: Sign Comparison

MOV R2, #-10

MOV R3, #10

CMN R2, R3      ; Compare signs

Lab 4: Loop Counter

MOV R4, #0

loop:

ADD R4, R4, #1

CMP R4, #10

BNE loop        ; Repeat loop

Equivalent C code:

x = 0;

while (x<10){

x++;}

Lab 5: Subroutine Call

BL subroutine  ; Call subroutine

...

subroutine:

PUSH {LR}

; Do something

BX LR          ; Return – calls to BL

B – branches unconditionally

BL – branch to address in LR

Lab 6: Set and Clear Bits

LDR R0, =GPIO_ADDR

LDR R1, [R0]

BIC R1, R1, #0x01   ; Set bit 0

STR R1, [R0]

BIC – bitwise clear

Value 1 * (~Value 2) = Result

R1 = 1011 1011   Mask = 0000 0001    ~Mask = 1111 1110

R1 * (~Mask) = 1011 1010

BIC: “set bits of R1 to 0 where Mask = 1” (bitwise AND with inverted mask)

Lab 7: SysTick Configuration

LDR R0, =STRELOAD ; correct syntax: NVIC_ST_RELOAD_R

MOV R1, #16000

STR R1, [R0]        ; Store reload value

Lab 8: GPIO Configuration

GPIO_DIR EQU 0x40025400

LDR R0, =GPIO_DIR

LDR R1, [R0]

ORR R1, R1, #0x02   ; Set bit 1

STR R1, [R0]

ORR – bitwise OR

#0x02 = 0010

ORR R1, R1, #2_0010 ; Sets bit 1 in R1 but leaves everything else alone

Lab 9: Interrupt Return

ISR_Handler:

PUSH {R4-R7}

; Handle interrupt

POP {R4-R7}

BX LR             ; Return from ISR to address in LR

Lab 10: Masking Bits

MOV R2, #0xFF

TST R2, #0x0F      ; Test lower nibble (least significant 4 bits)

Potential exam questions:

- Configuring the timer and knowing its registers

- Know about interrupts and its addresses

- The flag stuff from signed and unsigned arithmetic (CV flags)

- At least one code that does something, make sure to have the beginning part of the code included (the section with AREA and EXPORT Start)

- Understand DCB (1 byte = 8 bit), DCW (2 byte = 16 bit), DCD (4 byte = 32 bit)

- Index addressing

- Branching conceptual

- If-then-then-else (ITTE {cond})

- Conceptual equation about stack

- Write a program that checks if R0 is negative, zero, or positive, and branches to three different labels accordingly.

; Assume arbitrary value for R0

CMP R0, #0

BGT Positive

BLT Negative

BEQ Equal

- Simulate a nested loop: outer loop runs 5 times, inner loop runs 3 times. Use R0 and R1 as counters.

Start

MOV R0, #5 ; R0 = 5

OUTER

MOV R1, #3 ; R1 = 3

INNER

SUB R1, #1 ; Decrement R1

BNE INNER ; Repeat until Z=1

; ; ; ;

SUB R0, #1 ; Decrement R0

BNE OUTER ; Repeat until Z=1

Equivalent C code:

int x = 5;

while (x > 0){

x--;

int y = 3;

while (y > 0){

y--;

}

}

- Use TST to check if bits 3 and 5 of R4 are set. Branch to “bit_match” if both are set.

TST R4, #0x08

BEQ no_match

TST R4, #0x20

BEQ no_match

B bit_match

- Calculate and configure SysTick RELOAD for 1ms delay at 80MHz system clock.

;Initializing SysTick

;Step 1: Turn off SysTick timer

LDR R1, =NVIC_ST_CTRL_R

MOV R0, #0

STR R0, [R1] ;Disable counter

;Step 2: Loads reload register w/ counter start value

;RELOAD+1 = t*f = (1)(80*10^3) = 80,000

;RELOAD = 80,000 – 1 = 79,999 = 0x0001387F

LDR R2, =0x0001387F ;1ms delay

LDR R1, =NVIC_ST_RELOAD_R

STR R2, [R1]

;Step 3: Loading into STCURRENT clears counter & COUNT flag

LDR R1, =NVIC_ST_CURRENT_R

MOV R0, #0

STR R0, [R1]

;Step 4: Select system clock & turn on timer

LDR R1, =NVIC_ST_CTRL_R

MOV R0, #5 ;0101 = bit 0 enables counter & bit 2 sets system clock

STR R0, [R1]