Name:  _____________________________ (please print)

Signature: __________________________

ECE 3364 – Quiz #1

September 18, 2025

Do not open this quiz until you are told to begin.

- Print your name, and sign your name, at the top of this page.

- This quiz is closed book, closed notes.  You may use one 8.5” x 11” crib sheet, or its equivalent.  You may use a calculator. You should not use a cell phone, tablet computer, or laptop computer, as you work on this quiz.

- Show all work on these pages, and you may use both sides of each page.  Show all work necessary to complete the problem.  A solution without the appropriate work shown will receive no credit.  A solution which is not given in a reasonable order will lose credit.  You may separate the pages as you work.

- Show all units in solutions, intermediate results, and figures.  Units in the quiz will be included between square brackets.

- Do not use red ink.  Do not use red pencil.

- You will have 40 minutes to work on this quiz.

________________/20

Room for extra work

The circuit shown operates in steady-state.

The four loads are described as follows.

Load 1 is absorbing 2.73[kW] and 1.35[kVAR].

Load 2 is absorbing 3.57Ð-5.3°[kVA].

Load 3 is absorbing 12.86[kVA] at a power factor of 0.73 leading.

Load 4 is a 13.7[W] resistor in series with a 130[mF] capacitor.

- Find the reactive power delivered by the vB(t) voltage source.

- Find the steady-state value of iD(t).

Room for extra work

Solution follows.